Backend Formulas Reference

HazardMap backend mathematical formulas, thresholds, and decision rules. Generated from direct code inspection of the current repository.

# Quick Overview

| Area | What it controls |
| --- | --- |
| PGA and GMPE | Base shaking from magnitude, distance, depth, and regional model coefficients. |
| Soil amplification | Local site response using Vs30-derived NEHRP site classes. |
| MMI intensity | Human-readable shaking intensity derived from local PGA. |
| Impact aggregation | District/state summaries, risk categories, population affected, and damage score. |
| Contours | Smooth map polygons for PGA damage and MMI intensity. |

## 1. Surface Distance: Haversine

Source: backend/app/layers/pga/engine.py

a = sin²(Δlat / 2) + cos(lat1) * cos(lat2) * sin²(Δlon / 2)
surface_distance_km = 6371.0 * 2 * atan2(sqrt(a), sqrt(1 - a))

Why: Calculates epicenter-to-grid-cell distance on the curved Earth surface.

## 2. Hypocentral Distance

Source: backend/app/layers/pga/engine.py

R = sqrt(surface_distance² + depth²)
R = max(R, 1.0)

Why: Uses 3D distance from earthquake source and avoids log(0) in GMPE calculations.

## 3. PGA / GMPE

Source: backend/app/layers/pga/gmpe.py

M_adj = M - 6
ln(PGA) = c1 + c2*M_adj + c3*M_adj² + c4*R - C*ln(R)
PGA = exp(ln(PGA))

Why: Estimates base Peak Ground Acceleration in g from magnitude and hypocentral distance.

## 4. Regional GMPE Coefficients

Source: backend/app/layers/pga/gmpe.py

Himalayan:   c1=1.40, c2=0.50, c3=0.0, c4=-0.004,  C=1.00
Northeast:   c1=1.45, c2=0.55, c3=0.0, c4=-0.0045, C=1.00
Peninsular:  c1=1.30, c2=0.45, c3=0.0, c4=-0.003,  C=0.95

Why: Selects different attenuation behavior for different tectonic regions.

## 5. PGA Normalization

Source: backend/app/layers/pga/engine.py; backend/app/config.py

pga_normalized = clip(PGA / MAX_EXPECTED_PGA, 0, 1)
MAX_EXPECTED_PGA = 1.0g

Why: Converts PGA into a stable 0-1 score for map styling and scoring.

## 6. Vs30 to Site Class

Source: backend/app/soil/site_class.py

Vs30 > 1500        => A
760 < Vs30 <= 1500 => B
360 < Vs30 <= 760  => C
180 < Vs30 <= 360  => D
Vs30 <= 180        => E

Why: Classifies local ground conditions from shear-wave velocity.

## 7. Site Class to Soil Amplification

Source: backend/app/soil/amplification.py

A = 0.8x
B = 1.0x
C = 1.2x
D = 1.5x
E = 2.0x

Why: Models hard rock de-amplification and soft soil amplification.

## 8. Final / Local PGA

Source: backend/app/jobs/simulation_worker.py

pga_final = pga_base * soil_factor

Why: Produces local shaking after soil response; used for final heatmap and MMI.

## 9. PGA to MMI Intensity

Source: backend/app/jobs/simulation_worker.py

PGA_cm_s² = PGA_g * 980.665
MMI = 3.66 * log10(PGA_cm_s²) - 1.66
MMI = clip(MMI, 1.0, 10.0)

Why: Converts local PGA into Modified Mercalli Intensity using PGA in cm/s².

## 10. MMI to Roman Label

Source: backend/app/jobs/simulation_worker.py

< 2.5 => I-II
< 3.5 => III
< 4.5 => IV
< 5.5 => V
< 6.5 => VI
< 7.5 => VII
< 8.5 => VIII
< 9.5 => IX
>= 9.5 => X+

Why: Creates user-facing intensity labels for map panels and contours.

## 11. Generic Min-Max Normalization

Source: backend/app/layers/base.py

normalized = (value - min_value) / (max_value - min_value)
normalized = clip(normalized, 0, 1)

Why: Base utility for hazard layers that need relative scaling.

## 12. Soil Factor Clipping

Source: backend/app/layers/soil/engine.py

soil_factor_clipped = clip(soil_factor, 0.0, 2.0)

Why: Keeps site amplification multipliers within safe computation/display bounds.

## 13. Impact Aggregation

Source: backend/app/services/impact_aggregator.py

district max_pga = max(local PGA in district)
severe_cells += 1 if fused_hazard > 0.6
population affected += population if local PGA > 0.02g
avg_pga = total_pga / cell_count
damage_score = min(int(avg_pga * 50 + max_pga * 50), 100)

Why: Summarizes grid-cell shaking into district/state impact metrics.

## 14. Risk Category

Source: backend/app/services/impact_aggregator.py; backend/app/config.py

max_pga > 0.8 => EXTREME
max_pga > 0.6 => SEVERE
max_pga > 0.4 => HIGH
max_pga > 0.2 => MODERATE
else          => LOW

Why: Turns shaking into human-readable risk categories.

## 15. Contour Interpolation and Smoothing

Source: backend/app/services/contour_generator.py

grid_pga = LinearNDInterpolator / griddata over grid centroids
grid_pga = gaussian_filter(grid_pga, sigma=1.5)
grid_pga *= original_max_pga / blurred_max_pga

Why: Creates smooth contour heatmaps while preserving peak shaking after blur.

## 16. PGA Damage Contour Bands

Source: backend/app/config.py

0.014, 0.039, 0.092, 0.18, 0.34, 0.65, 1.24 g

Why: Defines low-to-extreme damage bands for PGA contours.

## 17. MMI Contour Bands

Source: backend/app/config.py

2.5, 3.5, 4.5, 5.5, 6.5, 7.5, 8.5, 9.5

Why: Defines MMI contour zones; I-II is omitted so very weak shaking leaves the basemap visible.

## 18. Event Relevance Filter

Source: backend/app/ingest/filter.py

magnitude >= AUTO_SIM_MIN_MAGNITUDE
depth_km <= AUTO_SIM_MAX_DEPTH_KM
point inside buffered India boundary

Why: Decides whether an ingested earthquake should trigger automatic simulation.

## 19. Tectonic Region Selection

Source: backend/app/layers/pga/regions.py

active margin buffer = 4.0 degrees
if inside active margin and lon > 89.0 => NORTHEAST
if inside active margin and lat < 25.0 => PENINSULAR
if inside active margin otherwise => HIMALAYA
else => PENINSULAR

Why: Chooses the regional GMPE model used for PGA estimation.